GarudaIII Release Notes

| Garuda III drivers Version |  | 3.2.5 |  |
| --- | --- | --- | --- |
| Release date |  | 23-OCT-26 |  |
| Sl. No | Component Name | Description | Version Info |
| 1 | Garuda3_0404_32.dll | J2534 Garuda III DLL | 3.0.2.5 R |
| 2 | Garuda III Firmware | Garuda III Firmware (REVC) | 3.2.5 C |
| 3 | Garuda III Firmware | Garuda III Firmware (REVA) | 3.2.5 A |
| 4 | GarudaConfig.exe | Configuration Utility to detect the Garuda Hardware | 3.0.0 |
| 5 | J2534SDK.exe | Sample Utility Application to test J2534 API's | 4.0.0 |
| 6 | GarudaIIIReflash.exe | Garuda Firmware Reflash Utility | 4.0.0 |
| Sl. No | Change Type (Feature / Issue/Improvement) | Description |  |
| 1 | Feature/Issue | - In CANFD periodic messages are now correctly padded to the nearest valid CANFD DLC - Fixed an issue where the CANFD Channel 2 message filter ID failed to reset to 1 after disconnecting and reconnecting the protocol |  |

| History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl. No | Date | Version No. | Change Type | Description |
| 1 | 14-OCT-26 | 3.2.4 | Feature/Issue | Implemented IOCTL call to select CANFD channel |
| 2 | 09-OCT-26 | 3.2.3 | Feature/Issue | CAN and CANFD Flashing Time Optimized – send bulk packet to handle Fixed No Response issue Developed CAN ISOTP driver Fixed Link Control issue – Baudrate change Resolved issue in dll logging |
| 3 | 20-AUG- 26 | 3.2.2 | Feature/Issue | Implemented Wi-Fi 5G support |
| 4 | 21-JUL-26 | 3.2.1 | Feature/Issue | Fix CAN FD Padding issue |
| 5 | 22-JUN-26 | 3.2.0 | Feature/Issue | Implement and integrated BT modules. Fix WiFi Consistency Issues |
| 6 | 11-JUN-26 | 3.1.9 | Issue | Implement and integrated BT modules. |
| 7 | 03-JUN-26 | 3.1.8 | Issue | Resolved issue: Fixed CANFD Data corruption, Fixed Channel 2 Communication issue, Introduced ECU Ping Option in SDK, Fixed CAN FD Baud rate configuration |
| 8 | 28-APR-26 | 3.1.7 | Issue | Resolved issue relating to stmin timing, Optimize flashing time using Reflash Utility. |
| 9 | 16-MAR-26 | 3.1.6 | Issue | Resolved issue relating to reading OBD2 Voltage. Unified the tgz file for both REVA and REVC. |
| 10 | 06-FEB-26 | 3.1.5 | Feature/Issue | Implemented/Added KWP protocol in Garuda lll. Implemented DoIP Periodic Messages, fixed support of 4K bytes for DoIP |
| 11 | 03-FEB-25 | 3.1.4 | Testing | Implemented/Added KWP protocol in Garuda lll. Implemented DoIP Periodic Messages, fixed support of 4K bytes for DoIP |
| 12 | 20-NOV-25 | 3.1.3 | Issue | Resolved an issue related to VIN fetching. |
| 13 | 15-NOV-25 | 3.1.2 | Issue | Resolved an issue related to buffer optimization. |
| 14 | 08-OCT-25 | 3.1.0 | Issue | Resolved an issue related to VIN fetching. |
| 15 | 20-AUG-25 | 3.0.9 | Issue | Resolved an issue related to RX buffer clearing. |
| 16 | 16-AUG-25 | 3.0.8 | Issue | Resolved the issue of HPCAP flashing NRC 72. |
| 17 | 16-AUG-25 | 3.0.8 | Improvement | Previously, the DoIP connection was taking up to 2 minutes, but it has now been reduced to 10 seconds. |
| 18 | 02-JULY-25 | 3.0.7 | Issue | Resolved the CAN FD communication issue in the write data function. |
| 19 | 24-JUNE-25 | 3.0.6 | Issue | Resolved the issue of HPCV flashing NRC 72. |
| 20 | 24-JUNE-25 | 3.0.6 | Issue | Resolved an issue where GarudaIII was incorrectly sending a CAN FD single frame as a CAN FD first frame. |
| 21 | 21-May-25 | 3.0.5 | Feature | Added New API: "PassThruReadSerailNo" to read the serial no. of the device |
| 22 | 21-Apr-25 | 3.0.4 | Issue | Resolved the problem with the RNDIS interface that was impacting USB communication, especially during flashing or reprogramming processes. |
| 23 | 21-Apr-25 | 3.0.4 | Feature | GarudaJ2534SDK Enhancements, After opening the device in GarudaJ2534SDK, it is now possible to read the firmware version and serial number without needing to uninstall the RNDIS driver in the device manger. |
| 24 | 17-Mar-25 | 3.0.3 | Issue | Resolved the issue of the Mida application crashing while using the "PassThruEnableRNDIS" API. |
| 25 | 06-Mar-25 | 3.0.2 | Feature | Added New API: "PassThruDOIPSendMsg" for larger bytes data transfer, solving the issue of larger bytes response handling. |
| 26 | 07-Feb-25 | 3.0.1 | Feature | Added New API: "PassThruEnableRNDIS" to bring-up the Ethernet interface |
| 27 | 07-Feb-25 | 3.0.1 | Feature | Added New API: "PassThruDisableRNDIS" to bring-down the Ethernet interface |